Control words used in the EDWRDS chemical equilibrium program, with most common keywords in bold:

CHP....CHAMBER PRESSURES        NMO....CANCEL MODIFY EXHAUST

CON....PRINT "CONFIDENTIAL"     NOU....CANCEL REGULAR OUTPUR

DET....DETONATION TEMP.         NPA....CANCEL PANIC BUTTON

DVE....DELTA VELOCITY           NSO....CANCEL SPECIAL OPT.

ELM....ADDITIONAL ELEMENTS      NTA....CANCEL TABULATION

END....TERMINATE PROGRAM        NTE....CANCEL TEMPREATURES

ENT....INPUT ENTHALPY           NTH....CANCEL THROAT CALC.

EPS....AREA RATIO CALC.         OPT....WEIGHT OPTIMIZATION

EXP....EXHAUST PRESSURES        OUT....PRINT REGULAR OUTPUT

ION....CONSIDER IONS            PAN....PANIC BUTTON

ITR....CHANGE ITERATION         PUN....PUNCH CARDS FOR PLUME

LIB....PROPELLANT LIBRARY       RED....READ STORED DATA TAPE

MIN....MINOR SPECIES            SEL....SELECT PROPELLANTS

MOD....MODIFY SPECIES           SOP....SPECIAL OPTIMIZATION

MOL....MOLE PROPORTIONS         TAB....TABULATE DATA

NCO....CANCEL "CONFIDENTIAL"    TEM....CHAMBER TEMPERATURE

NDF....CANCEL DETONATION        THR....THROAT SOLUTION

NEL....CANCEL ADD ELEMENTS      VOL....VOLUME PROPORTIONS

NEP....CANCEL AREA RATIOS       VOP....VOLUME OPTIMIZATION

NEX....SUPPRESS EXHAUST         WTS....WEIGHT PROPORTIONS

NIO....CANCEL IONS              XEQ....EXECUTE THE INPUT

NMI....CANCEL MINOR SPECIES     HEL... PRINT CONTROL WORDS

Notes:

When setting up the library (with the “LIB” command), an extra return follows the “LIB” command.  Then there are three lines of input:

- A name (which is just an identifier – this could be anything)

- The heat of formation and density.  The units on the input heat of formation are kcal/gmol.  The heat of formation is only needed if the output temperature is not specified (like an “H-P” calculation in NASA-CEA).  When working with a solid fuel, since a mol is vague, just make sure that the molecular formula matches the heat of formation, and then use the “WTS” command instead of the “MOL” command when specifying proportions of reactants.  The density is in g/cc, and is not needed unless you are calculating some of the rocket parameters, like specific impulse.

- The molecular formula of the input compound.  This has a special format.  There is a space before each element, then a comma, then a decimal number (with the decimal point) for the number of atoms of that element in the molecule.  A return should follow the last number.

After the 3 lines of the last species, an asterisk (“*”) indicates the end of the input species and exits the library setup.  An example library setup sequence is shown below {with comments in curly brackets -- do not include any comments in the input file}:

LIB	{start the library input routine}

{extra return; needed!}

AP	{Name you select for the compound}

-70.73,1.756	{Hf (kcal/gmol),  (g/cm3)}

H,4.0, N,1.0, O,4.0, CL,1.0	{H4.0N1.0O4.0Cl1.0, or NH4ClO4}

HTPB	{name of second species in library}

-168.57,0.88	{Hf (kcal/gmol),  (g/cm3)}

C,86.0, H,146.0, O,2.0	{C86.0H146.0O2.0}

*	{Asterisk to end the library input}

Other Notes

- To specify the inlet composition, use either MOL or WTS.  You can perform multiple calculations for different mole proportions or weight proportions by having extra lines of input, but an extra return is the signal for ending this option (see example input file).

- Chamber pressure (CHP) is in psia.

- The energy level can be specified in one of three ways:

  - TEM specifies the final temperature (K).

  - Use the heats of formation to do an adiabatic calculation, assuming that the heat of formation of each species is at the inlet temperature.  Do no specify TEM or ENT with this option.

  - ENT specifies the total enthalpy (kcal/100g?) of the final products, and then calculates a temperature to match.

- The NMI, NEX, and NTH keywords turn off many of the rocket calculations and simplify the output.